# Procedure for Downloading Configuration to the i32 Unit

- Open the iSeries SerComm program which can be downloaded from the Mirror Cooling and Defrost Status page.

- Set the com port to COM1 under ComSettings -> Port.  This needs to be performed every time the iSeries SerComm program is opened.

- Send the i32.txt configuration file to the i32 unit by executing: Transfer -> Send File.  This file is located on Mirror Cooling and Defrost Status page and also on the hard drive of the summit laptop computer under My Documents.

- You may have to perform a dummy read and/or write in order for the Send file command to work correctly.  This is done by executing: Configuration -> Read Config/Write Config.

- To change configuration values via the menu interface execute: Configuration -> Write Config.  This brings up the menu to change the setpoint, PID values, etc.