# To write Osprey container files with the LSI 11/23 system

- Turn off the TCS power.

- Remove the CPU card in Slot1 C, D and the memory card in Slot1 E, F.

- Put the Osprey QBUS card into the vacant memory slot (Slot 1 E, F).

- Turn on the TCS power.

- If the T5 machine is off, login under administrator with the password irtftcs!

- Start the GetDisk program under start/programs/osprey.

- Write disks images 0 and 1 respectively to: 
C:\Program Files\Strobe Data\Osprey\Images\”Filename.dx0”
C:\Program Files\Strobe Data\Osprey\Images\”Filename.dx1”
Full exact pathname must be type in.

- In the configuration file osprey.cnf located in the Osprey directory or accessible through the Osprey.exe program; add the appropriate pathnames to the container images.