TABLE OF CONTENTS

1	Test Overview	3

2	Equipment and Tools Required	3

3	Test Setup	3

4	Testing	3

TABLE OF REPORT TABLES

Table 1	Required Test Equipment	3

Table 2	Side1 Testing	3

Table 3	Side 2 Testing	3

# Test Overview

This test procedure is intended to present a simple, quick, DC only test to verify that the platform controller boards will operate correctly.

All fields with a yellow background are recordable data fields and must be completed.

# Equipment and Tools Required

The table below lists the equipment and tools required to complete this test procedure.

| Item # | Qty | Model / Part Number | Description |
| --- | --- | --- | --- |
| 1 | 1 | Fluke 179 (or equivalent) | Fluke Multimeter |
| 2 | 1 | - | +12V Supply |
| 3 | 1 | - | +1V Voltage Source (e.g. supply) |
| 4 | 1 | - | Wire with Minigrabber on one end. |
| 5 | REF | PC-1001 | Platform Controller Schematics |
| 6 | 2 | - | 50 +/-10% ohm leaded resistors |
| 7 | 1 | PC1000 | Platform Controller CCA (unit under test) |

T

Table 1	Required Test Equipment

# Test Setup

Follow these steps for the setup.

- Connect the +12V supply to TP8 “Vsupply”.

- Connect the +12V return to any black test point on the board “GND”.

- Connect the +1V return to any black test point on the board “GND”.

- Place the 50 ohm resistors in “Act_Test_1” and “Act_Test_2” sockets.

# Testing

There are two circuits (side 1 & 2) on this board.  Both will have to be tested.  For each side, measurements will be made with the +1V supply connected to “CMD_1” (or “CMD_2”) and with +1V disconnected.

| Test Point | Label | Voltage (V) (+1V Disconnected) | Spec (V) | Voltage (V) (+1V Connected to TP7) | Spec (V) | Tester Initials | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TP9 | “BR_RED_1” |  | 5.6 to 5.7 |  | 7.3 to 7.5 |  |  |
| TP10 | “BR_BLK_1” |  | 5.6 to 5.7 |  | 3.8 to 4.0 |  |  |
| Difference: TP9 – TP10 |  |  | 0.050 |  | 3.4 to 3.6 |  |  |

Table 2	Side1 Testing

| Test Point | Label | Voltage (V) (+1V Disconnected) | Spec (V) | Voltage (V) (+1V Connected to TP13) | Spec (V) | Tester Initials | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TP11 | “BR_RED_2” |  | 5.6 to 5.7 |  | 7.3 to 7.5 |  |  |
| TP12 | “BR_BLK_2” |  | 5.6 to 5.7 |  | 3.8 to 4.0 |  |  |
| Difference: TP11 – TP12 |  |  | 0.050 |  | 3.4 to 3.6 |  |  |

Table 3	Side 2 Testing

Did all tests pass? (circle one)	PASS	FAIL