UNIVERSITY OF HAWAII

INSTITUTE FOR ASTRONOMY

2680 Woodlawn Dr.

Honolulu, HI

96822

NASA Infrared Telescope Facility

TCS3 Electronics: Hazard Analysis and T3 Design

Fred Keske

July 30, 2026

Version 1

1	Hazards Overview	3

1.1	Potential Hazards	3

1.2	Response to Hazards	4

1.3	Enabling the Telescope	4

2	Overview of the T3 Electronics Board	4

2.1	Fault Logic	Error! Bookmark not defined.

2.2	Tachometer Circuit	4

2.3	Fault Logic	4

2.4	Watchdog Timer	5

3	TO Panel	5

Table 1Hazards	4

Figure 1 TO Panel Components	5

Figure 2 TO Panel Layout	6

# Hazards Overview

This document will attempt to present the potential hazards connected with operating the IRTF as it relates to the TCS and how the TCS3 and associated systems negate these hazards to put the telescope in a safe condition for operation, maintenance, and facility protection.  This document will mainly concentrate on the design of the T3 electronics that is associated with the TCS3 system.

## Potential Hazards

The major hazards areas are: ensuring the telescope is in a safe condition before enabling the system, bringing the telescope to a safe condition after loss of control, and disabling the telescope for maintenance.  The table below lists the known hazards and how they are handled by the TCS3 system.

| Hazard | Telescope Condition | TCS3 module(s) responding to condition |
| --- | --- | --- |
| Loss of Motor | Runaway | Software, T3 over speed circuit, stop and emergency safety switches to software and T3 circuit |
| Loss of Motor Amplifier | Runaway | Software, T3 over speed circuit, stop and emergency safety switches to software and T3 circuit |
| Loss of one Motor Amplifier Drive Signal | Runaway | Software, T3 over speed circuit, stop and emergency safety switches to software and T3 circuit |
| Loss of Motor Tachometer | Runaway | Software, T3 over speed circuit, stop and emergency safety switches to software and T3 circuit |
| Loss of Encoder | Runaway | Software, T3 over speed circuit, stop and emergency safety switches to software and T3 circuit |
| Jammed Telescope or dome | Slow or no movement, excessive amplifier current | Software, T3 over current and thermal disable circuit, amplifier thermal disable |
| Loss of TCS3 computer | Runaway or no movement | T3 watchdog circuit, over speed circuit, operator intervention |
| Loss of T3 electronics | Loss of fault detection | Software, operator intervention |
| Loss of motor controller card | Runaway or no movement | T3 motor control signal, T3 over speed circuit, stop and emergency safety switches to software and T3 circuit |
| Loss of safety switch | Runaway | Three levels of switches: slew, stop, emergency stop |
| Inadvertent movement of telescope or dome while maintenance is being performed | Personnel safety | T3 Keyed Lockout switch signal, software lockout signal |
| Platform not stowed | Platform, telescope damage | Software, operator intervention |
| Crane not stowed | Crane, telescope damage | Software, operator intervention |
| Telescope in slew speed nearing the end of axis | Runaway? | Software detection, slew safety switch to software |

Table 1Hazards

## Response to Hazards

In all cases when a fault condition exists, the motor amplifier drive inputs, the dome amplifier drive inputs, the system power relay and the brake release relay will be disabled by the T3 electronics board.

## Enabling the Telescope

To enable the system the operator must have the dome key switch in the enabled position, the telescope key switch in the enabled position, the emergency stop button in the released state and all external signals must be in their safe state; the external safety signals can be found on the top sheet of the T3 electronics schematic.  The TCS3 computer can also lockout the system so this signal must in its unlocked state (high).  The TCS motor controller card must then issue a low to high to low reset pulse to the T3 board which will activate the TCS enable signal that enables all five power amplifiers.  The TCS then issues the system power enable command which then enables the system power relay which supplies AC power to the motor, the telescope brakes, and the counterweight brakes.  The TCS then issues the brake release command which releases the telescope brakes.  The telescope is then ready for operation.

# Overview of the T3 Electronics Board

The philosophy behind the T3 electronics board is that it monitors safety critical signals during telescope operation and provides a second line of defense in the event that the TCS3 computer fails.  The board also contains circuitry for the HA and Dec tachometers.  The board is resident in the T3 electronics box along with the associated interface hardware for the motor controller board and draws its power from a power supply located in the box.

## Over speed and Over current

These circuits detect an over speed and over current condition for the HA and Dec axis.  The signals are brought to a variable voltage divider and compared to a precision ±1V reference.  The circuit is essentially the same as the TCS1 system with an improved voltage reference.

## Tachometer Circuit

This circuit provides the signal conditioning, filtering and summing of the tachometer signals. It is the same circuit used in the TCS1 system and was used in the Simulink simulation of the servo system.

## Fault Logic

The fault logic is contained in a separate project schematic to create the VHDL file.  The VHDL file is then used to create the configuration file for the XC9500 CPDL using the Xilinx toolset.  The external signal connections are noted on the top sheet of the T3 project.  As shown on the fault logic sheet, many signals are buffered with an inverter in order to drive the LEDs on the TO panel along with the Opto-22 digital inputs.

## Watchdog Timer

A watchdog timer is used to monitor the health of the TCS3 computer.  The TCS3 computer will output a 10 Hz, 50% duty cycle signal to the timer.  The timer is set to trigger if it does not detect a falling edge within 150ms.  A zero ohm resister may be removed to increase the timeout period to 600ms.

# TO Panel

The TO Panel contains the LED indicators for the safety switches and the system enable signals.  It also contains the system enable key switch, the over ride key switch, floor light switch, and the emergency stop button.   The override key switch allows the day crew to override the safety switches in order to move the telescope past the safety switches to the end of the axis for maintenance purposes.  The board also contains a joystick that is connected to an Opto-22 unit to allow for manual telescope positioning.  The LEDs and switches are shown in the figure 1 and a rough cut at the physical layout is shown in figure 2.

Figure 1 TO Panel Components

Figure 2 TO Panel Layout